ExpressPay Certificate Guide

V-1.0.1 2022-4-21

Shenzhen Xinguodu Technology Co.,LtdPublished

www.xinguodu.com

Copyright @ 2016-2026 Shenzhen New State Technology Co., Ltd.
China print and publish

This document may adjust the technical errors that may be included without notice. Modifications to this document will be released in the next release. This document prohibits any form of reproduction or propagation without permission.

Draft

| Version | Date | Editor | Description |
| --- | --- | --- | --- |
| V1.0.1 | 2022-04-20 | ShaoPu | First version |

Table of Content

Table of Content	2

1 Introduction	3

Terminal configuration	3

2 Terminal AID and CAPK configuration	4

AID	4

CAPK	5

Related EMV tags	13

3 Test cases	16

Global Expresspay EP4.0 3.0	16

AXP EP001	16

AXP EP003	19

AXP EP007	22

AXP EP008	24

AXP EP010	25

AXP EP017	27

AXP EP018	29

AXP EP019	32

AXP EP020	33

AXP EP023	36

AXP EP034	38

AXP EP084	40

AXP EP085	42

AXP EP088	44

AXP EP090	46

AXP EP091	48

AXP EP092	50

AXP EP097	52

4 FAQ	54

4.1 How to identify the card is DRL supported ?	54

4.2 How to set DRL for expresspay ?	55

4.3 how does kenrel perform this cvm processing?	56

4.4 When kernel will return SdkResult.Emv_Plz_See_Phone?	57

4.5 How to handle this SdkResult.Emv_Plz_See_Phone ?	57

1 Introduction

This document details the terminal capabilities, parameter settings, test casse usage, etc. Help developers perform Paywave certification.

Normally, the certification body will provide us with a TSE file. We just need to import the TSE file into Colis, and the test case will be automatically generated.

In this document, we will use a sample TSE file for generating test cases.

Terminal configuration

2 Terminal AID and CAPK configuration

AID

AID list:

a00000002501

AID detail for source code configuration:

{

"ddol": "9f3704",

"aid": "a00000002501",

"appVerNum": "0001",

"asi": 0,

"contactlessCvmLimit": 1000,

"contactlessFloorLimit": 1200,

"contactlessTransLimit": 1500,

"floorLimit": 0,

"maxTargetPercent": 99,

"onlinePinCap": 1,

"tacDefault": "DC50FC9800",

"tacDenial": "0010000000",

"tacOnline": "DE00FC9800",

"targetPercent": 99,

"threshold": 0,

"transLimit": 4000

}

CAPK

{

"arithInd": 1,

"capkIdx": 14,

"checkSum": "a7266abae64b42a3668851191d49856e17f8fbcd",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "aa94a8c6dad24f9ba56a27c09b01020819568b81a026be9fd0a3416ca9a71166ed5084ed91ced47dd457db7e6cbcd53e560bc5df48abc380993b6d549f5196cfa77dfb20a0296188e969a2772e8c4141665f8bb2516ba2c7b5fc91f8da04e8d512eb0f6411516fb86fc021ce7e969da94d33937909a53a57f907c40c22009da7532cb3be509ae173b39ad6a01ba5bb85",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 3,

"checkSum": "8708a3e3bbc1bb0be73ebd8d19d4e5d20166bf6c",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "b0c2c6e2a6386933cd17c239496bf48c57e389164f2a96bff133439ae8a77b20498bd4dc6959ab0c2d05d0723af3668901937b674e5a2fa92ddd5e78ea9d75d79620173cc269b35f463b3d4aaff2794f92e6c7a3fb95325d8ab95960c3066be548087bcb6ce12688144a8b4a66228ae4659c634c99e36011584c095082a3a3e3",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 15,

"checkSum": "a73472b3ab557493a9bc2179cc8014053b12bab4",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "c8d5ac27a5e1fb89978c7c6479af993ab3800eb243996fbb2ae26b67b23ac482c4b746005a51afa7d2d83e894f591a2357b30f85b85627ff15da12290f70f05766552ba11ad34b7109fa49de29dcb0109670875a17ea95549e92347b948aa1f045756de56b707e3863e59a6cbe99c1272ef65fb66cbb4cff070f36029dd76218b21242645b51ca752af37e70be1a84ff31079dc0048e928883ec4fadd497a719385c2bbbebc5a66aa5e5655d18034ec5",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 16,

"checkSum": "c729cf2fd262394abc4cc173506502446aa9b9fd",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "cf98dfedb3d3727965ee7797723355e0751c81d2d3df4d18ebab9fb9d49f38c8c4a826b99dc9dea3f01043d4bf22ac3550e2962a59639b1332156422f788b9c16d40135efd1ba94147750575e636b6ebc618734c91c1d1bf3edc2a46a43901668e0ffc136774080e888044f6a1e65dc9aaa8928dacbeb0db55ea3514686c6a732cef55ee27cf877f110652694a0e3484c855d882ae191674e25c296205bbb599455176fdd7bbc549f27ba5fe35336f7e29e68d783973199436633c67ee5a680f05160ed12d1665ec83d1997f10fd05bbdbf9433e8f797aee3e9f02a34228ace927abe62b8b9281ad08d3df5c7379685045d7ba5fcde58637",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 4,

"checkSum": "fdd7139ec7e0c33167fd61ad3cadbd68d66e91c5",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "d0f543f03f2517133ef2ba4a1104486758630dcfe3a883c77b4e4844e39a9bd6360d23e6644e1e071f196ddf2e4a68b4a3d93d14268d7240f6a14f0d714c17827d279d192e88931af7300727ae9da80a3f0e366aeba61778171737989e1ee309",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 101,

"checkSum": "894c5d08d4ea28bb79dc46cead998b877322f416",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "e53eb41f839ddfb474f272cd0cbe373d5468eb3f50f39c95bdf4d39fa82b98dabc9476b6ea350c0dce1cd92075d8c44d1e57283190f96b3537d9e632c461815ebd2baf36891df6bfb1d30fa0b752c43dca0257d35dff4ccfc98f84198d5152ec61d7b5f74bd09383bd0e2aa42298ffb02f0d79adb70d72243ee537f75536a8a8df962582e9e6812f3a0be02a4365400d",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 151,

"checkSum": "ebda522b631b3eb4f4cbfc0679c450139d2b69cd",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "e178ffe834b4b767af3c9a511f973d8e8505c5fcb2d3768075ab7cc946a955789955879aaf737407151521996dfa43c58e6b130eb1d863b85dc9ffb4050947a2676aa6a061a4a7ae1edb0e36a697e87e037517eb8923136875ba2ca1087cba7ec7653e5e28a0c261a033af27e3a67b64bba26956307ec47e674e3f8b722b3ae0498db16c7985310d9f3d117300d32b09",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 152,

"checkSum": "d4dba428cf11d45baeb0a35caea8007ad8ba8d71",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "d31a7094fb221cba6660fb975aafea80db7bb7eafd7351e748827ab62d4aeeccfc1787fd47a04699a02db00d7c382e80e804b35c59434c602389d691b9ccd51ed06be67a276119c4c10e2e40fc4eddf9df39b9b0bdee8d076e2a012e8a292af8efe18553470639c1a032252e0e5748b25a3f9ba4cfcee073038b061837f2ac1b04c279640f5bd110a9dc665ed2fa6828bd5d0fe810a892dee6b0e74ce8863bde08fd5fd61a0f11fa0d14978d8ced7dd3",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 153,

"checkSum": "94790d020f4f692d59289f36451872078005b63b",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "e1740074229fa0d228a9623581d7a322903fb89ba7686712e601fa8ab24a9789186f15b70ccbbe7421b1cb110d45361688135ffd0db15a3f516bb291d4a123ebf5a06fbf7e1ee6311b737dabb289570a7959d532b25f1da6758c84ddccadc049bc764c05391abd2cadeffa7e242d5dd06e56001f0e68151e3388074bd9330d6afa57cbf33946f531e51e0d4902ee235c756a905fb733940e6ec897b4944a5edc765705e2acf76c78ead78dd9b066df0b2c88750b8aee00c9b4d4091fa7338449da92dbfc908fa0781c0128c492db993c88ba8bb7cadfe238d477f2517e0e7e3d2b11796a0318ce2ad4da1db8e54ab0d94f109db9caeefbef",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 193,

"checkSum": "dc79d6b5fc879362299bc5a637dad2e0d99656b8",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "e69e319c34d1b4fb43aed4bd8bba6f7a8b763f2f6ee5ddf7c92579a984f89c4a9c15b27037764c58ac7e45efbc34e138e56ba38f76e803129a8ddeb5e1cc8c6b30cf634a9c9c1224bf1f0a9a18d79ed41ebcf1be78087ae8b7d2f896b1de8b7e784161a138a0f2169ad33e146d1b16ab595f9d7d98be671062d217f44eb68c68640c7d57465a063f6bac776d3e2dac61",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 194,

"checkSum": "8e748296359a7428f536adda8e2c037e2b697ef6",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "b875002f38ba26d61167c5d440367604ad38df2e93d8ee8da0e8d9c0cf4cc5788d11dea689e5f41d23a3da3e0b1fa5875ae25620f5a6bccee098c1b35c691889d7d0ef670eb8312e7123fcc5dc7d2f0719cc80e1a93017f944d097330edf945762fee62b7b0ba0348228dbf38d4216e5a67a7ef74f5d3111c44aa31320f623cb3c53e60966d6920067c9e082b746117e48e4f00e110950ca54da3e38e5453bd5544e3a6760e3a6a42766ad2284e0c9af",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 195,

"checkSum": "12f1790cb0273dc73c6e70784bc24c12e8db71f6",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "b93182abe343dfbf388c71c4d6747dcdec60367fe63cfaa942d7d323e688d0832836548bf0edff1edeeb882c75099ff81a93fa525c32425b36023ea02a8899b9bf7d7934e86f997891823006ceaa93091a73c1fde18abd4f87a22308640c064c8c027685f1b2db7b741b67ab0de05e870481c5f972508c17f57e4f833d63220f6ea2cfbb878728aa5887de407d10c6b8f58d46779ecec1e2155487d52c78a5c03897f2bb580e0a2bbde8ea2e1c18f6aaf3eb3d04c3477deab88f150c8810fd1ef8eb0596866336fe2c1fbc6bec22b4fe5d885647726db59709a505f75c49e0d8d71bf51e4181212be2142ab2a1e8c0d3b7136cd7b7708e4d",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 199,

"checkSum": "6221e0c726bac8f8ac25f8f93b811d1ffd4c131c",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "cd237e34e0299de48f1a2c94f478fe972896011e1ca6ab462b68fe0f6109c9a97c2dbeea65932cde0625138b9f162b92979daab019d3b5561d31eb2d4f09f12f927ea8f740ce0e87154965505e2272f69042b15d57ccc7f771919123978283b3cce524d9715207bf5f5ad369102176f0f7a78a6deb2bff0edce165f3b14f14d0035b2756861fe03c43396ed002c894a3",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 200,

"checkSum": "33bd7a059fab094939b90a8f35845c9dc779bd50",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "bf0cfced708fb6b048e3014336ea24aa007d7967b8aa4e613d26d015c4fe7805d9db131ced0d2a8ed504c3b5ccd48c33199e5a5bf644da043b54dbf60276f05b1750fab39098c7511d04babc649482ddcf7cc42c8c435bab8dd0eb1a620c31111d1aaaf9af6571eebd4cf5a08496d57e7abdbb5180e0a42da869ab95fb620eff2641c3702af3be0b0c138eaef202e21d",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 201,

"checkSum": "8e8dff443d78cd91de88821d70c98f0638e51e49",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "b362db5733c15b8797b8ecee55cb1a371f760e0bedd3715bb270424fd4ea26062c38c3f4aaa3732a83d36ea8e9602f6683eecc6baff63dd2d49014bde4d6d603cd744206b05b4bad0c64c63ab3976b5c8caaf8539549f5921c0b700d5b0f83c4e7e946068baaab5463544db18c63801118f2182efcc8a1e85e53c2a7ae839a5c6a3cabe73762b70d170ab64afc6ca482944902611fb0061e09a67acb77e493d998a0ccf93d81a4f6c0dc6b7df22e62db",

"rid": "a000000025"

}, {

"arithInd": 1,

"capkIdx": 202,

"checkSum": "6bda32b1aa171444c7e8f88075a74fbfe845765f",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "c23ecbd7119f479c2ee546c123a585d697a7d10b55c2d28bef0d299c01dc65420a03fe5227ecdecb8025fbc86eebc1935298c1753ab849936749719591758c315fa150400789bb14fadd6eae2ad617da38163199d1bad5d3f8f6a7a20aef420adfe2404d30b219359c6a4952565ccca6f11ec5be564b49b0ea5bf5b3dc8c5c6401208d0029c3957a8c5922cbde39d3a564c6debb6bd2aef91fc27bb3d3892beb9646dce2e1ef8581effa712158aaec541c0bbb4b3e279d7da54e45a0acc3570e712c9f7cdf985cfafd382ae13a3b214a9e8e1e71ab1ea707895112abc3a97d0fcb0ae2ee5c85492b6cfd54885cdd6337e895cc70fb3255e3",

"rid": "a000000025"

}

Note: capkIdx is int type in above list. For example 239(int) = 0xEF(Hexadecimal)

Related EMV tags

Terminal Type – Modified

Send in GPO command

9F6E : Enhanced Contactless Reader Capabilities

Send with unmodified 9F35 in GPO command.

82: application interchange profile

Returned from GPO response

9F70: Card Interface Capabilities

Returned from read record command

9F71: Mobile CVM Results

Returned from GPO command

3 Test cases

Global Expresspay EP4.0 3.0

AXP EP001

Enter amount 6.01

Test Report:

Android logcat:

AXP EP003

Enter amount 12.01

Test Report:

Android Logcat:

AXP EP007

Test Report:

Android Logcat:

AXP EP008

Enter amount 2.00

Test Report:

Android Logcat:

AXP EP010

Enter Amount 5.01

Test Report:

Android logcat:

AXP EP017

Enter amount 12.02

Test Report:

Android logcat:

AXP EP018

Enter amount 12.03

Test report:

Android logcat:

AXP EP019

Enter amount 2.02

Test Report:

Android logcat:

AXP EP020

Enter amount 11.02

Test report:

Android logcat:

AXP EP023

Enter amount 10.02:

Test report :

Android logcat:

AXP EP034

Enter refund amount 1.10:

Test Report:

Android logcat:

AXP EP084

Enter amount 12.81

Test report:

Android logcat:

AXP EP085

Enter amount 12.82:

Test report:

Android logcat:

AXP EP088

Enter amount 12.85

Test report:

Android logcat:

AXP EP090

Enter amount 13.31

Test report:

Android logcat:

AXP EP091

Enter amount 1.06

Test report:

Android logcat:

Note: request ARQC casued by SDA selected even floow limit not exceeded.

AXP EP092

Enter amount 13.33

Test report:

Android logcat:

AXP EP097

Enter amount 12.50

Test report:

Android logcat:

4 FAQ

4.1 How to identify the card is DRL supported ?

Check tag 9F70 byte 2.

4.2 How to set DRL for expresspay ?

DynamicReaderLimitEntity defaultDynamicReaderLimitEntity = new DynamicReaderLimitEntity();

defaultDynamicReaderLimitEntity.setAppProgID(new byte[]{(byte) 0xFF});

defaultDynamicReaderLimitEntity.setDrlSupport(false);

defaultDynamicReaderLimitEntity.setReaderCVMReqLimit(ByteUtils.hexString2ByteArray("000000001000"));

defaultDynamicReaderLimitEntity.setReaderContactlessTransLimit(ByteUtils.hexString2ByteArray("000000001500"));

defaultDynamicReaderLimitEntity.setReaderContactlessFloorLimit(ByteUtils.hexString2ByteArray("000000001200"));

List<DynamicReaderLimitEntity> dynamicReaderLimitEntities = new ArrayList<>();

//used for EP094,EP095

DynamicReaderLimitEntity dynamicReaderLimitEntity = new DynamicReaderLimitEntity();

dynamicReaderLimitEntity.setAppProgID(new byte[]{(byte) 0x06});

dynamicReaderLimitEntity.setReaderCVMReqLimit(ByteUtils.hexString2ByteArray("000000000200"));

dynamicReaderLimitEntity.setReaderContactlessTransLimit(ByteUtils.hexString2ByteArray("000000000700"));

dynamicReaderLimitEntity.setReaderContactlessFloorLimit(ByteUtils.hexString2ByteArray("000000000400"));

dynamicReaderLimitEntities.add(dynamicReaderLimitEntity);

//used for EP096

dynamicReaderLimitEntity = new DynamicReaderLimitEntity();

dynamicReaderLimitEntity.setAppProgID(new byte[]{(byte) 0x0B});

dynamicReaderLimitEntity.setReaderCVMReqLimit(ByteUtils.hexString2ByteArray("000000000200"));

dynamicReaderLimitEntity.setReaderContactlessTransLimit(ByteUtils.hexString2ByteArray("000000000300"));

dynamicReaderLimitEntity.setReaderContactlessFloorLimit(ByteUtils.hexString2ByteArray("000000000100"));

dynamicReaderLimitEntities.add(dynamicReaderLimitEntity);

//default drl will replace all expresspay aid limits even card doesn’t support drl

int ret = emvHandler2.setDynamicReaderLimitListForExpressPay(defaultDynamicReaderLimitEntity, dynamicReaderLimitEntities);

Log.d(TAG, "setDynamicReaderLimitListForExpressPay : " + ret);

4.3 how does kenrel perform this cvm processing?

We assume that card AIP byte 1 bit 5 indicates cardholder verification supported has been set and terminal configure mobile cvm, online pin, signature supported on 9F6E.

If entered amount is smaller than cvm limit then 9F6E byet 3 bit 7 will be set to 0 and CVM result should be CVMR_NA

Else If entered amount is greater or equal to cvm limit then 9F6E byte 3 bit 7 will be set 1 and perform following processing:

- check whetehr 9F71 mobile cvm result returned from GPO and byte1 indicate mobile CVM performed successfully, finally the cvm result should be CVMR_CDCVM

- check tag 8E and identify mutually supported cvm. If online pin selected then cvm result shall be CVMR_ONLINEPIN, if signature selected then cvm result shall be CVMR_SIGNATURE

4.4 When kernel will return SdkResult.Emv_Plz_See_Phone?

If entered amount is greater then cvm limit then 9F6E byte 3 bit 7 will be set to 1 indicated cvm required, but 9F71 mobile cvm result returned from GPO indicatged no cvm performed then kernel will return this -8027 guide you to see phone handle this mobile cvm. Finally search card again and startemvprocess to finsih this transaction.

4.5 How to handle this SdkResult.Emv_Plz_See_Phone EP020?