Android POS EMV L3 Certification Guide

Version 1.0

Contents

Version History	5

1. Introduction	6

1.1. Objective	6

2. VISA	7

2.1. Terminal Configuration	7

2.2. Test AID & CAPK	8

2.2.1. AID	8

2.2.2. CAPK	9

2.3. Test cases	13

2.3.1. VISA.TC.1002a	13

2.3.2. VISA.TC.1011a	14

2.3.3. VISA.TC.1018a	15

2.3.4. VISA.TC.3103	16

2.3.5. VISA.TC.3116	17

3. MasterCard	18

3.1. Terminal Configuration	18

3.2. Test AID & CAPK	19

3.2.1. AID	19

3.2.2. CAPK	21

3.3. Test cases	27

3.3.1. MCD03.Test.07.Scenario.01	27

3.3.2. MCD19.Test.04.Scenario.01	28

3.3.3. MCD55.Test.01.Scenario.01	29

3.3.4. MCM01.Test.01.Scenario.01	30

3.3.5. MCM02.Test.02.Scenario.01	31

3.3.6. MSI01.Test.16.Scenario.04	32

3.3.7. MSI04.Test.01.Scenario.01	33

3.3.8. MSI92.Test.01.Scenario.01	34

3.3.9. MSI94.Test.03.Scenario.01	35

3.3.10. USM01.Test.20.Scenario.01	36

3.3.11. USM05.Test.03.Scenario.01	37

3.3.12. USM50.Test.01.Scenario.01	38

3.3.13. USM92.Test.01.Scenario.01	39

4. AMEX	40

4.1. Terminal Configuration	40

4.2. Test AID & CAPK	41

4.2.1. AID	41

4.2.2. CAPK	42

4.3. Test cases	43

4.3.1. AXP CTL-EP001	43

4.3.2. AXP EP001	44

4.3.3. AXP EP003	44

4.3.4. AXP EP018	44

5. Discover	45

5.1. Terminal Configuration	45

5.2. Test AID & CAPK	46

5.2.1. AID	46

5.2.2. CAPK	48

5.3. Test cases	50

5.3.1. DGN_DPAS_L3_CL_086	50

6. JCB	51

6.1. Terminal Configuration	51

6.2. Test AID & CAPK	52

6.2.1. AID	52

6.2.2. CAPK	53

6.3. Test cases	56

6.3.1. 212_01	56

6.3.2. 213_01	57

6.3.3. 213_02	58

6.3.4. 214_02	59

7. UnionPay	60

7.1. Terminal Configuration	60

7.2. Test AID & CAPK	61

7.2.1. AID	61

7.2.2. CAPK	63

7.3. Test cases	66

7.4. Matching Currency	66

7.5. Unmatching Currency	67

- Version History

| Version | Date | Comment | Author |
| --- | --- | --- | --- |
| 1.0 | 2025-11-10 | Initial Version | xxxx |

Introduction

Objective

This document details the terminal capabilities, parameter settings, test case usage, etc. Help developers perform EMV L3 certifications.

Normally, the certification body will provide us with a TSE file. We just need to import the TSE file into Colis, and the test case will be automatically generated.

In this document, we will use a sample TSE file for generating test cases.

VISA

Terminal Configuration

| Terminal type | Pos |
| --- | --- |
| Terminal capability | Offline with online capability |
| Online Pin | Support |
| Signature | Support |
| Contact EMV | Support |
| qVSDC | Support |
| MSD | Do not support MSD for PAYWAVE |
| Offline Data Authentication | Support |
| Offline Data Authentication for Online Transaction | Support |
| Electron AID | Support |
| Interlink AID | Support |
| Extended Selection | Support |
| TTQ B2b6 is set(switch interface when CVM is required) | Support |
| Terminal Contactless Floor Limit | 0.00 |
| Terminal Contactless CVM Limit | 15.00 |
| Terminal Contactless Trans Limit | 50.00 |

Test AID & CAPK

AID

AID list:

A000000003

AID detail for source code Configuration:

{

"ddol": "9f3704",

"aid": "a000000003",

"appVerNum": "008c",

"asi": 0,// partial match flag. asi=0, partial match;asi=1, full match

"contactlessCvmLimit": 1500,

"contactlessFloorLimit": 0,

"contactlessTransLimit": 5000,

"floorLimit": 0,

"maxTargetPercent": 0,

"onlinePinCap": 1,

"tacDefault": "DC4000A800",

"tacDenial": "0010000000",

"tacOnline": "DC4004F800",

"targetPercent": 0,

"threshold": 0,

"transLimit": 4000

}

CAPK

{

"arithInd": 1,

"capkIdx": 1,

"checkSum": "d34a6a776011c7e7ce3aec5f03ad2f8cfc5503cc",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "c696034213d7d8546984579d1d0f0ea519cff8deffc429354cf3a871a6f7183f1228da5c7470c055387100cb935a712c4e2864df5d64ba93fe7e63e71f25b1e5f5298575ebe1c63aa617706917911dc2a75ac28b251c7ef40f2365912490b939bca2124a30a28f54402c34aeca331ab67e1e79b285dd5771b5d9ff79ea630b75",

"rid": "a000000003"

}, {

"arithInd": 1,

"capkIdx": 7,

"checkSum": "b4bc56cc4e88324932cbc643d6898f6fe593b172",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a89f25a56fa6da258c8ca8b40427d927b4a1eb4d7ea326bbb12f97ded70ae5e4480fc9c5e8a972177110a1cc318d06d2f8f5c4844ac5fa79a4dc470bb11ed635699c17081b90f1b984f12e92c1c529276d8af8ec7f28492097d8cd5becea16fe4088f6cfab4a1b42328a1b996f9278b0b7e3311ca5ef856c2f888474b83612a82e4e00d0cd4069a6783140433d50725f",

"rid": "a000000003"

}, {

"arithInd": 1,

"capkIdx": 8,

"checkSum": "20d213126955de205adc2fd2822bd22de21cf9a8",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "d9fd6ed75d51d0e30664bd157023eaa1ffa871e4da65672b863d255e81e137a51de4f72bcc9e44ace12127f87e263d3af9dd9cf35ca4a7b01e907000ba85d24954c2fca3074825ddd4c0c8f186cb020f683e02f2dead3969133f06f7845166aceb57ca0fc2603445469811d293bfefbafab57631b3dd91e796bf850a25012f1ae38f05aa5c4d6d03b1dc2e568612785938bbc9b3cd3a910c1da55a5a9218ace0f7a21287752682f15832a678d6e1ed0b",

"rid": "a000000003"

}, {

"arithInd": 1,

"capkIdx": 9,

"checkSum": "1ff80a40173f52d7d27e0f26a146a1c8ccb29046",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "9d912248de0a4e39c1a7dde3f6d2588992c1a4095afbd1824d1ba74847f2bc4926d2efd904b4b54954cd189a54c5d1179654f8f9b0d2ab5f0357eb642feda95d3912c6576945fab897e7062caa44a4aa06b8fe6e3dba18af6ae3738e30429ee9be03427c9d64f695fa8cab4bfe376853ea34ad1d76bfcad15908c077ffe6dc5521ecef5d278a96e26f57359ffaeda19434b937f1ad999dc5c41eb11935b44c18100e857f431a4a5a6bb65114f174c2d7b59fdf237d6bb1dd0916e644d709ded56481477c75d95cdd68254615f7740ec07f330ac5d67bcd75bf23d28a140826c026dbde971a37cd3ef9b8df644ac385010501efc6509d7a41",

"rid": "a000000003"

}, {

"arithInd": 1,

"capkIdx": 153,

"checkSum": "4abffd6b1c51212d05552e431c5b17007d2f5e6d",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "ab79fcc9520896967e776e64444e5dcdd6e13611874f3985722520425295eea4bd0c2781de7f31cd3d041f565f747306eed62954b17edaba3a6c5b85a1de1beb9a34141af38fcf8279c9dea0d5a6710d08db4124f041945587e20359bab47b7575ad94262d4b25f264af33dedcf28e09615e937de32edc03c54445fe7e382777",

"rid": "a000000003"

}, {

"arithInd": 1,

"capkIdx": 149,

"checkSum": "ee1511cec71020a9b90443b37b1d5f6e703030f6",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "be9e1fa5e9a803852999c4ab432db28600dcd9dab76dfaaa47355a0fe37b1508ac6bf38860d3c6c2e5b12a3caaf2a7005a7241ebaa7771112c74cf9a0634652fbca0e5980c54a64761ea101a114e0f0b5572add57d010b7c9c887e104ca4ee1272da66d997b9a90b5a6d624ab6c57e73c8f919000eb5f684898ef8c3dbefb330c62660bed88ea78e909aff05f6da627b",

"rid": "a000000003"

}, {

"arithInd": 1,

"capkIdx": 146,

"checkSum": "429c954a3859cef91295f663c963e582ed6eb253",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "996af56f569187d09293c14810450ed8ee3357397b18a2458efaa92da3b6df6514ec060195318fd43be9b8f0cc669e3f844057cbddf8bda191bb64473bc8dc9a730db8f6b4ede3924186ffd9b8c7735789c23a36ba0b8af65372eb57ea5d89e7d14e9c7b6b557460f10885da16ac923f15af3758f0f03ebd3c5c2c949cba306db44e6a2c076c5f67e281d7ef56785dc4d75945e491f01918800a9e2dc66f60080566ce0daf8d17ead46ad8e30a247c9f",

"rid": "a000000003"

}, {

"arithInd": 1,

"capkIdx": 148,

"checkSum": "c4a3c43ccf87327d136b804160e47d43b60e6e0f",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "acd2b12302ee644f3f835abd1fc7a6f62cce48ffec622aa8ef062bef6fb8ba8bc68bbf6ab5870eed579bc3973e121303d34841a796d6dcbc41dbf9e52c4609795c0ccf7ee86fa1d5cb041071ed2c51d2202f63f1156c58a92d38bc60bdf424e1776e2bc9648078a03b36fb554375fc53d57c73f5160ea59f3afc5398ec7b67758d65c9bff7828b6b82d4be124a416ab7301914311ea462c19f771f31b3b57336000dff732d3b83de07052d730354d297bec72871dccf0e193f171aba27ee464c6a97690943d59bdabb2a27eb71ceebdafa1176046478fd62fec452d5ca393296530aa3f41927adfe434a2df2ae3054f8840657a26e0fc617",

"rid": "a000000003"

}, {

"arithInd": 1,

"capkIdx": 87,

"checkSum": "251a5f5de61cf28b5c6e2b5807c0644a01d46ff5",

"expireDate": "20491231",

"exponent": "010001",

"hashInd": 1,

"modulus": "942b7f2ba5ea307312b63df77c5243618acc2002bd7ecb74d821fe7bdc78bf28f49f74190ad9b23b9713b140ffec1fb429d93f56bdc7ade4ac075d75532c1e590b21874c7952f29b8c0f0c1ce3aeedc8da25343123e71dcf86c6998e15f756e3",

"rid": "a000000003"

},{

"arithInd": 1,

"capkIdx": 16,

"checkSum": "833B1947778036B6D759FCE3F618DDEB2749372C",

"expireDate": "20491231",

"exponent": "03",

"hashInd": 1,

"modulus": "9F2701C0909CCBD8C3ED3E071C69F776160022FF3299807ED7A035ED5752770E232D56CC3BE159BD8F0CA8B59435688922F406F55C75639457BBABEFE9A86B2269EF223E34B91AA6DF2CCAD03B4AD4B443D61575CA960845E6C69040101E231D9EF811AD99B0715065A0E661449C41B4B023B7716D1E4AFF1C90704E55AE1225",

"rid": "a000000003"

},

Note: CapkIdx is int type in above list. For example 239(int) = 0xEF(Hexadecimal)

Test cases

VISA.TC.1002a

Question

Ensure online approval for a VCPS 2.2 baseline card and TTQ settings are correct.

Solution:

Set the value of Tag 9F66 as the case required, reference value “36E04000”

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("9F66"),ConvertUtils.hexString2Bytes("36E04000"));

VISA.TC.1011a

Question

The EMV onFinish callback will return -8027(Please See Phone).

Solution:

Ensure online approval for a VCPS 2.2 baseline card and TTQ settings are correct.

This is the See Phone test cases.

Application needs to display “please check your Phone” message or equivalent message. Then call search card API to search the contactless card again

Example code for Android device

slotTypes.add(CardSlotTypeEnum.RF);
cardReader.searchCard(slotTypes, 20, this);

VISA.TC.1018a

Question

The EMV onFinish callback will return -8030

- Solution:

Set the byte 1 bit 8 of Tag 9F66 to “MSD not Support”. Reference value “36E04000”

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("9F66"),ConvertUtils.hexString2Bytes("36E04000"));

VISA.TC.3103

Question

This case tests SCA-insert card. The first contactless transaction sends to host, and host return specific response code 1A, application receive this special code needs to let user insert card, and start new EMV flow(contact) again

VISA.TC.3116

Question

This case tests SCA-input online PIN. The first contactless transaction sends to host, and host return specific response code 70, application receive this special code needs to let user enter online PIN, then send the transaction to the host again (EMV data is same with previous transaction)

MasterCard

Terminal Configuration

| Terminal type | Attended Pos |
| --- | --- |
| Terminal capability | Offline with online capability |
| Online Pin | Support |
| Signature | Support |
| Contact EMV | Support |
| MSD | Do not support MSD for PAYPASS |
| Maestro Card | Support |
| RRP | Support |
| Terminal Contactless Floor Limit | 0.00 |
| Terminal Contactless CVM Limit | 500.00 |
| Terminal Contactless Trans Limit | 500000.00 |

Test AID & CAPK

AID

AID list:

A0000000041010

A00000000410

A00000000430

A0000000043060

AID detail for source code configuration:

{

"DDOL": "9f3704",

"aid": "a0000000041010",

"appVerNum": "0002",

"asi": 0,

"contactlessCvmLimit": 50000,

"contactlessFloorLimit": 20000,

"contactlessTransLimit": 50000000,

"floorLimit": 0,

"maxTargetPercent": 99,

"onlinePinCap": 1,

"tacDefault": "fc50b8a000",

"tacDenial": "0000000000",

"tacOnline": "fc50808800",

"targetPercent": 99,

"threshold": 0,

"transLimit": 4000

}, {

"DDOL": "9f3704",

"aid": "a00000000410",

"appVerNum": "0002",

"asi": 0,

"contactlessCvmLimit": 10000,

"contactlessFloorLimit": 20000,

"contactlessTransLimit": 30000,

"floorLimit": 0,

"maxTargetPercent": 0,

"onlinePinCap": 1,

"tacDefault": "fc50aca000",

"tacDenial": "0400000000",

"tacOnline": "f850acf800",

"targetPercent": 0,

"threshold": 0,

"transLimit": 4000

}, {

"DDOL": "9f3704",

"aid": "a00000000430",

"appVerNum": "0002",

"asi": 0,

"contactlessCvmLimit": 10000,

"contactlessFloorLimit": 20000,

"contactlessTransLimit": 30000,

"floorLimit": 0,

"maxTargetPercent": 0,

"onlinePinCap": 1,

"tacDefault": "fc50aca000",

"tacDenial": "0400000000",

"tacOnline": "f850acf800",

"targetPercent": 0,

"threshold": 0,

"transLimit": 4000

}, {

"DDOL": "9f3704",

"aid": "a0000000043060",

"appVerNum": "0002",

"asi": 0,

"contactlessCvmLimit": 50000,

"contactlessFloorLimit": 0,

"contactlessTransLimit": 50000000,

"floorLimit": 0,

"maxTargetPercent": 99,

"onlinePinCap": 1,

"tacDefault": "fc50bca000",

"tacDenial": "0000800000",

"tacOnline": "fc50bcf800",

"targetPercent": 99,

"threshold": 0,

"transLimit": 4000

}

CAPK

{

"arithInd": 1,

"capkIdx": 0,

"checkSum": "ec0a59d35d19f031e9e8cbec56db80e22b1de130",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "9c6be5adb10b4be3dce2099b4b210672b89656eba091204f613ecc623bedc9c6d77b660e8baeea7f7ce30f1b153879a4e36459343d1fe47acdbd41fcd710030c2ba1d9461597982c6e1bdd08554b726f5eff7913ce59e79e357295c321e26d0b8be270a9442345c753e2aa2acfc9d30850602fe6cac00c6ddf6b8d9d9b4879b2826b042a07f0e5ae526a3d3c4d22c72b9eaa52eed8893866f866387ac05a1399",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 2,

"checkSum": "33408b96c814742ad73536c72f0926e4471e8e47",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a99a6d3e071889ed9e3a0c391c69b0b804fc160b2b4bdd570c92dd5a0f45f53e8621f7c96c40224266735e1ee1b3c06238ae35046320fd8e81f8ceb3f8b4c97b940930a3ac5e790086dad41a6a4f5117ba1ce2438a51ac053eb002aed866d2c458fd73359021a12029a0c043045c11664fe0219ec63c10bf2155bb2784609a106421d45163799738c1c30909bb6c6fe52bbb76397b9740ce064a613ff8411185f08842a423ead20edffbff1cd6c3fe0c9821479199c26d8572cc8afff087a9c3",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 3,

"checkSum": "5addf21d09278661141179cbeff272ea384b13bb",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "c2490747fe17eb0584c88d47b1602704150adc88c5b998bd59ce043edebf0ffee3093ac7956ad3b6ad4554c6de19a178d6da295be15d5220645e3c8131666fa4be5b84fe131ea44b039307638b9e74a8c42564f892a64df1cb15712b736e3374f1bbb6819371602d8970e97b900793c7c2a89a4a1649a59be680574dd0b60145",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 4,

"checkSum": "381a035da58b482ee2af75f4c3f2ca469ba4aa6c",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a6da428387a502d7ddfb7a74d3f412be762627197b25435b7a81716a700157ddd06f7cc99d6ca28c2470527e2c03616b9c59217357c2674f583b3ba5c7dcf2838692d023e3562420b4615c439ca97c44dc9a249cfce7b3bfb22f68228c3af13329aa4a613cf8dd853502373d62e49ab256d2bc17120e54aedced6d96a4287acc5c04677d4a5a320db8bee2f775e5fec5",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 5,

"checkSum": "ebfa0d5d06d8ce702da3eae890701d45e274c845",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "b8048abc30c90d976336543e3fd7091c8fe4800df820ed55e7e94813ed00555b573feca3d84af6131a651d66cff4284fb13b635edd0ee40176d8bf04b7fd1c7bacf9ac7327dfaa8aa72d10db3b8e70b2ddd811cb4196525ea386acc33c0d9d4575916469c4e4f53e8e1c912cc618cb22dde7c3568e90022e6bba770202e4522a2dd623d180e215bd1d1507fe3dc90ca310d27b3efccd8f83de3052cad1e48938c68d095aac91b5f37e28bb49ec7ed597",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 6,

"checkSum": "f910a1504d5ffb793d94f3b500765e1abcad72d9",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "cb26fc830b43785b2bce37c81ed334622f9622f4c89aae641046b2353433883f307fb7c974162da72f7a4ec75d9d657336865b8d3023d3d645667625c9a07a6b7a137cf0c64198ae38fc238006fb2603f41f4f3bb9da1347270f2f5d8c606e420958c5f7d50a71de30142f70de468889b5e3a08695b938a50fc980393a9cbce44ad2d64f630bb33ad3f5f5fd495d31f37818c1d94071342e07f1bec2194f6035ba5ded3936500eb82dfda6e8afb655b1ef3d0d7ebf86b66dd9f29f6b1d324fe8b26ce38ab2013dd13f611e7a594d675c4432350ea244cc34f3873cba06592987a1d7e852adc22ef5a2ee28132031e48f74037e3b34ab747f",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 241,

"checkSum": "d8e68da167ab5a85d8c3d55ecb9b0517a1a5b4bb",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a0dcf4bde19c3546b4b6f0414d174dde294aabbb828c5a834d73aae27c99b0b053a90278007239b6459ff0bbcd7b4b9c6c50ac02ce91368da1bd21aaeadbc65347337d89b68f5c99a09d05be02dd1f8c5ba20e2f13fb2a27c41d3f85cad5cf6668e75851ec66edbf98851fd4e42c44c1d59f5984703b27d5b9f21b8fa0d93279fbbf69e090642909c9ea27f898959541aa6757f5f624104f6e1d3a9532f2a6e51515aead1b43b3d7835088a2fafa7be7",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 243,

"checkSum": "a69ac7603daf566e972dedc2cb433e07e8b01a9a",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "98f0c770f23864c2e766df02d1e833dff4ffe92d696e1642f0a88c5694c6479d16db1537bfe29e4fdc6e6e8afd1b0eb7ea0124723c333179bf19e93f10658b2f776e829e87daeda9c94a8b3382199a350c077977c97aff08fd11310ac950a72c3ca5002ef513fccc286e646e3c5387535d509514b3b326e1234f9cb48c36ddd44b416d23654034a66f403ba511c5efa3",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 245,

"checkSum": "c2239804c8098170be52d6d5d4159e81ce8466bf",

"expireDate": "20201231",

"exponent": "010001",

"hashInd": 1,

"modulus": "a6e6fb72179506f860ccca8c27f99cecd94c7d4f3191d303bbee37481c7aa15f233ba755e9e4376345a9a67e7994bdc1c680bb3522d8c93eb0ccc91ad31ad450da30d337662d19ac03e2b4ef5f6ec18282d491e19767d7b24542dfdeff6f62185503532069bbb369e3bb9fb19ac6f1c30b97d249eee764e0bac97f25c873d973953e5153a42064bbfabfd06a4bb486860bf6637406c9fc36813a4a75f75c31cca9f69f8de59adecef6bde7e07800fcbe035d3176af8473e23e9aa3dfee221196d1148302677c720cfe2544a03db553e7f1b8427ba1cc72b0f29b12dfef4c081d076d353e71880aadff386352af0ab7b28ed49e1e672d11f9",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 246,

"checkSum": "502909ed545e3c8dbd00ea582d0617fee9f6f684",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a25a6bd783a5ef6b8fb6f83055c260f5f99ea16678f3b9053e0f6498e82c3f5d1e8c38f13588017e2b12b3d8ff6f50167f46442910729e9e4d1b3739e5067c0ac7a1f4487e35f675bc16e233315165cb142bfdb25e301a632a54a3371ebab6572deebaf370f337f057ee73b4ae46d1a8bc4da853ec3cc12c8cbc2da18322d68530c70b22bdac351dd36068ae321e11abf264f4d3569bb71214545005558de26083c735db776368172fe8c2f5c85e8b5b890cc682911d2de71fa626b8817fccc08922b703869f3baeac1459d77cd85376bc36182f4238314d6c4212fbdd7f23d3",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 247,

"checkSum": "eeb0dd9b2477bee3209a914cdba94c1c4a9bded9",

"expireDate": "20201231",

"exponent": "010001",

"hashInd": 1,

"modulus": "94ea62f6d58320e354c022addcf0559d8cf206cd92e869564905ce21d720f971b7aea374830ebe1757115a85e088d41c6b77cf5ec821f30b1d890417bf2fa31e5908ded5fa677f8c7b184ad09028fdde96b6a6109850aa800175eabcdbbb684a96c2eb6379dfea08d32fe2331fe103233ad58dcdb1e6e077cb9f24eaec5c25af",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 248,

"checkSum": "f06ecc6d2aaebf259b7e755a38d9a9b24e2ff3dd",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a1f5e1c9bd8650bd43ab6ee56b891ef7459c0a24fa84f9127d1a6c79d4930f6db1852e2510f18b61cd354db83a356bd190b88ab8df04284d02a4204a7b6cb7c5551977a9b36379ca3de1a08e69f301c95cc1c20506959275f41723dd5d2925290579e5a95b0df6323fc8e9273d6f849198c4996209166d9bfc973c361cc826e1",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 249,

"checkSum": "336712dcc28554809c6aa9b02358de6f755164db",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a99a6d3e071889ed9e3a0c391c69b0b804fc160b2b4bdd570c92dd5a0f45f53e8621f7c96c40224266735e1ee1b3c06238ae35046320fd8e81f8ceb3f8b4c97b940930a3ac5e790086dad41a6a4f5117ba1ce2438a51ac053eb002aed866d2c458fd73359021a12029a0c043045c11664fe0219ec63c10bf2155bb2784609a106421d45163799738c1c30909bb6c6fe52bbb76397b9740ce064a613ff8411185f08842a423ead20edffbff1cd6c3fe0c9821479199c26d8572cc8afff087a9c3",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 250,

"checkSum": "5bed4068d96ea16d2d77e03d6036fc7a160ea99c",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a90fcd55aa2d5d9963e35ed0f440177699832f49c6bab15cdae5794be93f934d4462d5d12762e48c38ba83d8445deaa74195a301a102b2f114eada0d180ee5e7a5c73e0c4e11f67a43ddab5d55683b1474cc0627f44b8d3088a492ffaadad4f42422d0e7013536c3c49ad3d0fae96459b0f6b1b6056538a3d6d44640f94467b108867dec40faaecd740c00e2b7a8852d",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 254,

"checkSum": "9a295b05fb390ef7923f57618a9fda2941fc34e0",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a653eac1c0f786c8724f737f172997d63d1c3251c44402049b865bae877d0f398cbfbe8a6035e24afa086befde9351e54b95708ee672f0968bcd50dce40f783322b2aba04ef137ef18abf03c7dbc5813aeaef3aa7797ba15df7d5ba1cbaf7fd520b5a482d8d3fee105077871113e23a49af3926554a70fe10ed728cf793b62a1",

"rid": "a000000004"

}, {

"arithInd": 1,

"capkIdx": 239,

"checkSum": "21766ebb0ee122afb65d7845b73db46bab65427a",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a191cb87473f29349b5d60a88b3eaee0973aa6f1a082f358d849fddff9c091f899eda9792caf09ef28f5d22404b88a2293eebbc1949c43bea4d60cfd879a1539544e09e0f09f60f065b2bf2a13ecc705f3d468b9d33ae77ad9d3f19ca40f23dcf5eb7c04dc8f69eba565b1ebcb4686cd274785530ff6f6e9ee43aa43fdb02ce00daec15c7b8fd6a9b394baba419d3f6dc85e16569be8e76989688efea2df22ff7d35c043338deaa982a02b866de5328519ebbcd6f03cdd686673847f84db651ab86c28cf1462562c577b853564a290c8556d818531268d25cc98a4cc6a0bdfffda2dcca3a94c998559e307fddf915006d9a987b07ddaeb3b",

"rid": "a000000004"

}

Note: CapkIdx is int type in above list. For example 239(int) = 0xEF(Hexadecimal)

Test cases

MCD03.Test.07.Scenario.01

Question

In First GEN AC, TVR Byte 3, bit 8(Cardholder verification was not successful) = 1

In First GEN AC, CVM Results = 3F0001

Solution:

Set the value of DF8118 to “40” to support Online PIN only.

Example code for Android device

emvHandler.setTlv(ConvertUtils.hexString2Bytes(""),ConvertUtils.hexString2Bytes("40"));

MCD19.Test.04.Scenario.01

Question

To ensure that the terminal correctly transmits Tag 9F33 (Terminal Capabilities)

Solution:

Set the byte 2 of Tag 9F33 to “08”.

Example code for Android Device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("9F33"),ConvertUtils.hexString2Bytes("E008C8"));

MCD55.Test.01.Scenario.01

Question

To ensure that the terminal correctly manages DE 48 SE 22 (Multi-Purpose Merchant Indicator) in Single Tap and PIN request messages

Solution:

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF8119"),ConvertUtils.hexString2Bytes("08"));

MCM01.Test.01.Scenario.01

Question

The terminal informs the cardholder that the transaction is approved

Emv fallback return -8028

Solution:

Set the value of Tag DF811B to “20”.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF811B"),ConvertUtils.hexString2Bytes("20"));

MCM02.Test.02.Scenario.01

Question

To ensure that the terminal correctly manages mag-stripe mode contactless transactions

Solution:

Set the value of Tag DF811B to disable MSD, reference value “80”.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF811B"),ConvertUtils.hexString2Bytes("80"));

MSI01.Test.16.Scenario.04

Question

In First GEN AC, CVM Results = 1F0300 or 1F0302

Received: ’42 03 00’

Expected: ‘1F 03 02’

Solutions:

Set the value of Tag 9F33, Tag DF8118 and Tag DF8119 to support “No CVM” only.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("9F33"),ConvertUtils.hexString2Bytes("E048C8"));
emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF8118"),ConvertUtils.hexString2Bytes("10"));
emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF8119"),ConvertUtils.hexString2Bytes("10"));

MSI04.Test.01.Scenario.01

Question

In First GEN AC, CVM Results = 3F0002

Solutions:

Set the value of Tag 9F33, Tag DF8118 and Tag DF8119 to support “No CVM” only.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("9F33"),ConvertUtils.hexString2Bytes("E008C8"));
emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF8118"),ConvertUtils.hexString2Bytes("08"));
emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF8119"),ConvertUtils.hexString2Bytes("08"));

MSI92.Test.01.Scenario.01

Question

Card-Terminal validation (contactless)

The terminal does not send any command to the card beyond the GetPoresponse

Received: ‘07’

Expected: ‘05’

Solution:

Set the value of Tag DF811B to disable EMV Mode, reference value “A0”.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF811B"),ConvertUtils.hexString2Bytes("A0"));

MSI94.Test.03.Scenario.01

Question

In First GEN AC, the terminal requests an AAC

Received: ‘10’

Expected: ‘00’

Solution:

Set the value of Tag DF8121 to match the expected value of Tag 95, then the terminal will request AAC, reference value “fc50b8a000”.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF8121"),ConvertUtils.hexString2Bytes("fc50b8a000"));

USM01.Test.20.Scenario.01

Question

In First GEN AC, CVM Results = 3F0001

Solution:

Set the value of Tag DF8118 and Tag DF8119 to disable Online PIN.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF8118"),ConvertUtils.hexString2Bytes("00"));

emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF8118"),ConvertUtils.hexString2Bytes("08"));

USM05.Test.03.Scenario.01

Objectives

Make sure the <!--Kernel Configuration, b8-only EMV mode, b7-only mag-stripe mode, b6-On device cardholder verification supported(for mobile)-->

Solution:

Set the value of Tag to DF811B to “80”.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF811B"),ConvertUtils.hexString2Bytes("80"));

USM50.Test.01.Scenario.01

Question

In First GEN AC, TVR Byte 5, bit 1-2 = RRP Performed

Recevied: ‘00’

Expected: ‘10’

Solution:

Set the value of Tag DF811B to “30”.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF811B"),ConvertUtils.hexString2Bytes("30"));

USM92.Test.01.Scenario.01

Question

The terminal does not send any command to the card beyond the GetPoresponse

Received: ‘07’

Expected: ’05’

Solution:

Set the value of Tag DF811B to disable EMV mode contactless, reference value “A0”.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("DF811B"),ConvertUtils.hexString2Bytes("A0"));

AMEX

Terminal Configuration

| Terminal type | Attended Pos |
| --- | --- |
| Terminal capability | Offline with online capability |
| Online Pin | Support |
| Signature | Support |
| Contact EMV | Support |
| Terminal Contactless Floor Limit | 12.00 |
| Terminal Contactless CVM Limit | 10.00 |
| Terminal Contactless Trans Limit | 15.00 |

Test AID & CAPK

AID

AID list:

a00000002501

AID detail for source code configuration:

{
  "ddol":"9f3704",
  "aid":"a00000002501",
  "appVerNum":"0001",
  "asi":0,
  "contactlessCvmLimit":1000,
  "contactlessFloorLimit":1200,
  "contactlessTransLimit":1500,
  "floorLimit":0,
  "maxTargetPercent":99,
  "onlinePinCap":1,
  "tacDefault":"DC50FC9800",
  "tacDenial":"0010000000",
  "tacOnline":"DE00FC9800",
  "targetPercent":99,
  "threshold":0,
  "transLimit":4000
}

CAPK

{
  "arithInd":1,
  "capkIdx":200,
  "checkSum":"33bd7a059fab094939b90a8f35845c9dc779bd50",
  "expireDate":"20201231",
  "exponent":"03",
  "hashInd":1,
  "modulus":

"bf0cfced708fb6b048e3014336ea24aa007d7967b8aa4e613d26d015c4fe7805d9db131ced0d2a8ed504c3b5ccd48c33199e5a5bf644da043b54dbf60276f05b1750fab39098c7511d04babc649482ddcf7cc42c8c435bab8dd0eb1a620c31111d1aaaf9af6571eebd4cf5a08496d57e7abdbb5180e0a42da869ab95fb620eff2641c3702af3be0b0c138eaef202e21d",
  "rid":"a000000025"
}

Note: CapkIdx is int type in above list. For example 200 = 0xC38(exadecimal)

Test cases

AXP CTL-EP001

Solution

When CDA fail, it normal that the card expired, or you haven’t add the card required CAPK,

1. You can check the card’s expired date, you can get it from track2

2. You can find the tag 8F CAPK index value, then check your CAPK parameter file whether exist this CAPK?

AXP EP001

Question

In First GEN AC, TVR - Byte 1 bit 3 = ‘0’ indicates "CDA passed or not performed".

In First GEN AC Response, Tag 9F27(CID) = ’40' indicates ‘TC’.

Solution:

Add the index C8 CAPK value in your CAPK’s parameter file.

AXP EP003

Question

In First GEN AC, TVR - Byte 3 bit 8 = ‘1’ indicates "Transaction exceeds floor limit".

In First GEN AC Response, Tag 9F27(CID) = ’80' indicates ‘ARQC’.

Solution:

Please kindly make sure that your transaction amount more than contactless floor limit.

AXP EP018

Question

In First GEN AC, TVR - Byte 3 bit 8 = ‘0’ indicates "Cardholder verification successful or not performed".

Solution:

Please kindly set the 9F6D value for C0 in beforeGPO function.

Example code for Android device

emvHandler2.setTlv(ByteUtils.hexString2ByteArray("9F6D"),ByteUtils.hexString2ByteArray("C0"));

Discover

Terminal Configuration

| Terminal type | POS |
| --- | --- |
| Terminal capability | Offline with online capability |
| Online Pin | Support |
| Offline enciphered PIN | Support |
| Signature | Support |
| No CVM | Support |
| Specification | D-PAS Connect 2.x |
| Terminal Contactless Floor Limit | 0.00 |
| Terminal Contactless CVM Limit | 100.00 |
| Terminal Contactless Trans Limit | 20000.00 |

Test AID & CAPK

AID

AID list:

A0000001523010

A0000001524010

AID detail for source code configuration:

{

"ddol": "9f3704",

"aid": "a0000001523010",

"appVerNum": "0001",

"asi": 0,

"contactlessCvmLimit": 10000,

"contactlessFloorLimit": 0,

"contactlessTransLimit": 30000,

"floorLimit": 0,

"maxTargetPercent": 99,

"onlinePinCap": 1,

"tacDefault": "dc00002000",

"tacDenial": "0010000000",

"tacOnline": "fce09cf800",

"targetPercent": 99,

"threshold": 0,

"transLimit": 4000

}, {

"ddol": "9f3704",

"aid": "a0000001524010",

"appVerNum": "0001",

"asi": 0,

"contactlessCvmLimit": 10000,

"contactlessFloorLimit": 0,

"contactlessTransLimit": 30000,

"floorLimit": 0,

"maxTargetPercent": 99,

"onlinePinCap": 1,

"tacDefault": "dc00002000",

"tacDenial": "0010000000",

"tacOnline": "fce09cf800",

"targetPercent": 99,

"threshold": 0,

"transLimit": 4000

}

CAPK

{

"arithInd": 1,

"capkIdx": 90,

"checkSum": "cc9585e8e637191c10fcecb32b5ae1b9d410b52d",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "edd8252468a705614b4d07de3211b30031aedb6d33a4315f2cff7c97db918993c2dc02e79e2ff8a2683d5bbd0f614bc9ab360a448283ef8b9cf6731d71d6be939b7c5d0b0452d660cf24c21c47cac8e26948c8eed8e3d00c016828d642816e658dc2cfc61e7e7d7740633befe34107c1fb55dea7faaea2b25e85bed948893d07",

"rid": "a000000152"

}, {

"arithInd": 1,

"capkIdx": 91,

"checkSum": "4dc5c6cab6ae96974d9dc8b2435e21f526bc7a60",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "d3f45d065d4d900f68b2129afa38f549ab9ae4619e5545814e468f382049a0b9776620da60d62537f0705a2c926dbead4ca7cb43f0f0dd809584e9f7efbda3778747bc9e25c5606526fab5e491646d4dd28278691c25956c8fed5e452f2442e25edc6b0c1aa4b2e9ec4ad9b25a1b836295b823eddc5eb6e1e0a3f41b28db8c3b7e3e9b5979cd7e079ef024095a1d19dd",

"rid": "a000000152"

}, {

"arithInd": 1,

"capkIdx": 92,

"checkSum": "60154098cbba350f5f486ca31083d1fc474e31f8",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "833f275fcf5ca4cb6f1bf880e54dcfeb721a316692cafeb28b698caecafa2b2d2ad8517b1efb59ddefc39f9c3b33ddee40e7a63c03e90a4dd261bc0f28b42ea6e7a1f307178e2d63fa1649155c3a5f926b4c7d7c258bca98ef90c7f4117c205e8e32c45d10e3d494059d2f2933891b979ce4a831b301b0550cdae9b67064b31d8b481b85a5b046be8ffa7bdb58dc0d7032525297f26ff619af7f15bcec0c92bcdcbc4fb207d115aa65cd04c1cf982191",

"rid": "a000000152"

}, {

"arithInd": 1,

"capkIdx": 93,

"checkSum": "b51ec5f7de9bb6d8bce8fb5f69ba57a04221f39b",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "ad938ea9888e5155f8cd272749172b3a8c504c17460efa0bed7cbc5fd32c4a80fd810312281b5a35562800cdc325358a9639c501a537b7ae43df263e6d232b811acdb6dde979d55d6c911173483993a423a0a5b1e1a70237885a241b8eebb5571e2d32b41f9cc5514df83f0d69270e109af1422f985a52cce04f3df269b795155a68ad2d6b660ddcd759f0a5da7b64104d22c2771ece7a5ffd40c774e441379d1132faf04cdf55b9504c6dce9f61776d81c7c45f19b9efb3749ac7d486a5ad2e781fa9d082fb2677665b99fa5f1553135a1fd2a2a9fbf625ca84a7d736521431178f13100a2516f9a43ce095b032b886c7a6ab126e203be7",

"rid": "a000000152"

}

Note: CapkIdx is int type in above list. For example 239(int) = 0xEF(Hexadecimal)

Test cases

DGN_DPAS_L3_CL_086

Question

The card shall return Signed Dynamic Application Data (Tag 9F4B) in GPO response

Solution:

Set the DPAS version to 2.x.

Example code for Android device

//Set the DPAS version to 2.x before calling emvHandler.emvProcess

if(emvTransData.getCardType()== CardType.CTLS) {

emvTransDataEntity.setEmvEntryModeEnum(EmvEntryModeEnum.EMV_ENTRY_MODE_CONTACTLESS);

DPASTransDataEntity dpasTransDataEntity = new DPASTransDataEntity();

dpasTransDataEntity.setDpasVersion(DPASVersionEnum.DPAS_VERSION_2);

emvTransDataEntity.setDpasTransDataEntity(dpasTransDataEntity);

}else{

emvTransDataEntity.setEmvEntryModeEnum(EmvEntryModeEnum.EMV_ENTRY_MODE_CONTACT);

}

//Set the TTQ in onTransInitBeforeGPO callback

byte[]kernelTTQ=emvHandler2.getTlv(ByteUtils.hexString2ByteArray("9F66"), EmvDataSourceEnum.FROM_KERNEL);

kernelTTQ[0] = (byte) (kernelTTQ[0]|0x01);

emvHandler2.setTlv(ByteUtils.hexString2ByteArray("9F66"), kernelTTQ);

JCB

Terminal Configuration

| Terminal type | POS |
| --- | --- |
| Terminal capability | Offline with online capability |
| Online Pin | Support |
| Signature | Support |
| Offline Enciphered PIN | Support |
| Terminal Floor Limit | 0.00 |
| Terminal Contactless CVM Limit | 100.00 |
| Terminal Contactless Trans Limit | 20000.00 |

Test AID & CAPK

AID

AID list:

A0000006351010

AID detail for source code configuration:

{

"ddol": "9f3704",

"aid": "a0000006351010",

"appVerNum": "0096",

"asi": 0,

"contactlessCvmLimit": 10000,

"contactlessFloorLimit": 20000,

"contactlessTransLimit": 999999999,

"floorLimit": 0,

"maxTargetPercent": 0,

"onlinePinCap": 1,

"tacDefault": "fcf8fcf800",

"tacDenial": "0000000000",

"tacOnline": "fcf8fcf800",

"targetPercent": 0,

"threshold": 0,

"transLimit": 10000

}

CAPK

{

"arithInd": 1,

"capkIdx": 9,

"checkSum": "4410c6d51c2f83adfd92528fa6e38a32df048d0a",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "b72a8fef5b27f2b550398fdcc256f714bad497ff56094b7408328cb626aa6f0e6a9df8388eb9887bc930170bcc1213e90fc070d52c8dcd0ff9e10fad36801fe93fc998a721705091f18bc7c98241cadc15a2b9da7fb963142c0ab640d5d0135e77ebae95af1b4fefadcf9c012366bdda0455c1564a68810d7127676d493890bd",

"rid": "a000000065"

}, {

"arithInd": 1,

"capkIdx": 16,

"checkSum": "c75e5210cbe6e8f0594a0f1911b07418cadb5bab",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "99b63464ee0b4957e4fd23bf923d12b61469b8fff8814346b2ed6a780f8988ea9cf0433bc1e655f05efa66d0c98098f25b659d7a25b8478a36e489760d071f54cdf7416948ed733d816349da2aadda227ee45936203cbf628cd033aaba5e5a6e4ae37fbacb4611b4113ed427529c636f6c3304f8abdd6d9ad660516ae87f7f2ddf1d2fa44c164727e56bbc9ba23c0285",

"rid": "a000000065"

}, {

"arithInd": 1,

"capkIdx": 18,

"checkSum": "874b379b7f607dc1caf87a19e400b6a9e25163e8",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "adf05cd4c5b490b087c3467b0f3043750438848461288bfefd6198dd576dc3ad7a7cfa07dba128c247a8eab30dc3a30b02fcd7f1c8167965463626feff8ab1aa61a4b9aef09ee12b009842a1aba01adb4a2b170668781ec92b60f605fd12b2b2a6f1fe734be510f60dc5d189e401451b62b4e06851ec20ebff4522aacc2e9cdc89bc5d8cde5d633cfd77220ff6bbd4a9b441473cc3c6fefc8d13e57c3de97e1269fa19f655215b23563ed1d1860d8681",

"rid": "a000000065"

}, {

"arithInd": 1,

"capkIdx": 20,

"checkSum": "c0d15f6cd957e491db56dcdd1ca87a03ebe06b7b",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "aeed55b9ee00e1eceb045f61d2da9a66ab637b43fb5cdbdb22a2fbb25be061e937e38244ee5132f530144a3f268907d8fd648863f5a96fed7e42089e93457adc0e1bc89c58a0db72675fbc47fee9ff33c16ade6d341936b06b6a6f5ef6f66a4edd981df75da8399c3053f430eca342437c23af423a211ac9f58eaf09b0f837de9d86c7109db1646561aa5af0289af5514ac64bc2d9d36a179bb8a7971e2bfa03a9e4b847fd3d63524d43a0e8003547b94a8a75e519df3177d0a60bc0b4bab1ea59a2cbb4d2d62354e926e9c7d3be4181e81ba60f8285a896d17da8c3242481b6c405769a39d547c74ed9ff95a70a796046b5eff36682dc29",

"rid": "a000000065"

}, {

"arithInd": 1,

"capkIdx": 8,

"checkSum": "dd36d5896228c8c4900742f107e2f91fe50bc7ee",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "b74670dad1dc8983652000e5a7f2f8b35dfd083ee593e5ba895c95729f2bade9c8abf3dd9ce240c451c6ceffc768d83cbac76abb8fea58f013c647007cff7617bac2ae3981816f25cc7e5238ef34c4f02d0b01c24f80c2c65e7e7743a4fa8e23206a23ece290c26ea56db085c5c5eae26292451fc8292f9957be8ff20fad53e5",

"rid": "a000000065"

}, {

"arithInd": 1,

"capkIdx": 15,

"checkSum": "2a1b82de00f5f0c401760adf528228d3ede0f403",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "9efbadde4071d4ef98c969eb32af854864602e515d6501fde576b310964a4f7c2ce842abefafc5dc9e26a619bcf2614fe07375b9249befa09cfee70232e75ffd647571280c76ffca87511ad255b98a6b577591af01d003bd6bf7e1fce4dfd20d0d0297ed5eca25de261f37efe9e175fb5f12d2503d8cfb060a63138511fe0e125cf3a643afd7d66dcf9682bd246ddea1",

"rid": "a000000065"

}, {

"arithInd": 1,

"capkIdx": 17,

"checkSum": "d9fd62c9dd4e6de7741e9a17fb1ff2c5db948bcb",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a2583aa40746e3a63c22478f576d1efc5fb046135a6fc739e82b55035f71b09beb566edb9968dd649b94b6dedc033899884e908c27be1cd291e5436f762553297763daa3b890d778c0f01e3344cecdfb3ba70d7e055b8c760d0179a403d6b55f2b3b083912b183adb7927441bed3395a199eefe0debd1f5fc3264033da856f4a8b93916885bd42f9c1f456aab8cfa83ac574833eb5e87bb9d4c006a4b5346bd9e17e139ab6552d9c58bc041195336485",

"rid": "a000000065"

}, {

"arithInd": 1,

"capkIdx": 19,

"checkSum": "54cfae617150dfa09d3f901c9123524523ebedf3",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a3270868367e6e29349fc2743ee545ac53bd3029782488997650108524fd051e3b6eaca6a9a6c1441d28889a5f46413c8f62f3645aaeb30a1521eef41fd4f3445bfa1ab29f9ac1a74d9a16b93293296cb09162b149bac22f88ad8f322d684d6b49a12413fc1b6ac70ededb18ec1585519a89b50b3d03e14063c2ca58b7c2ba7fb22799a33bcde6afcbeb4a7d64911d08d18c47f9bd14a9fad8805a15de5a38945a97919b7ab88efa11a88c0cd92c6ee7dc352ab0746abf13585913c8a4e04464b77909c6bd94341a8976c4769ea6c0d30a60f4ee8fa19e767b170df4fa80312dba61db645d5d1560873e2674e1f620083f30180bd96ca589",

"rid": "a000000065"

}

Note: CapkIdx is int type in above list. For example 239(int) = 0xEF(Hexadecimal)

Test cases

212_01

Question

Tag 9F53 B1b7 is set to 1 when Signature for contactless transactions is supported in the ICS.

- Solution:

Set the value of Tag 9F52 and Tag 9F53 as the case required.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

//9F52 to set magstripe mode not support

byte[] tagff8178 = emvHandler.getTlv(new byte[]{(byte) 0xff, (byte) 0x81,(byte) 0x78}, EmvDataSourceEnum.FROM_KERNEL);

tagff8178[0] = setBitValue(tagff8178[0], 2, (byte) 0);

emvHandler.setTlv(ConvertUtils.hexString2Bytes("ff8178"),tagff8178);

tagff8178 = emvHandler.getTlv(new byte[]{(byte) 0xff, (byte) 0x81,(byte) 0x78}, EmvDataSourceEnum.FROM_KERNEL);

//9F53 byte2 bit8 set to 0, disable issuer update is prohibited

emvHandler.setJcbContactlessTIP(new byte[]{(byte) 0x73, (byte) 0x00,(byte) 0x00});

213_01

Question

Is the Merchant Category Code (tag 9F15) set to zero in the CDOL?

Is the Interface Device Serial Number (tag 9F1E) set to zero in the CDOL?

Solution:

Set the value of Tag 9F15 and Tag 9F1E as the case required.

Example code for Android device

//You can set the tag in onConfirmCardNo callback

byte[] aid = emvHandler.getTlv(new byte[]{0x4F}, EmvDataSourceEnum.FROM_KERNEL);

if(ConvertUtils.bytes2HexString(aid, "").toUpperCase().contains("A000000065")){

emvHandler.setTlv(ConvertUtils.hexString2Bytes("9F15"), ConvertUtils.hexString2Bytes("0000"));

emvHandler.setTlv(ConvertUtils.hexString2Bytes("9F1E"), ConvertUtils.hexString2Bytes("0000000000000000"));

}

213_02

Question

Is the Merchant Category Code (tag 9F15) set to zero in the CDOL?

Is the Interface Device Serial Number (tag 9F1E) set to zero in the CDOL?

Solution:

Set the value of Tag 9F15 and Tag 9F1E as case required.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("9F15"), ConvertUtils.hexString2Bytes("0000"));

emvHandler.setTlv(ConvertUtils.hexString2Bytes("9F1E"), ConvertUtils.hexString2Bytes("0000000000000000"));

214_02

Question

Is the Acquirer Identifier (tag 9F01) set to zero in the CDOL?

Is the Transaction Currency Exponent (tag 5F36) set to zero in the CDOL?

Terminal Interchange Profile (tag 9F53), B1b7 (Signature Supported) = 1

Solution:

Set the value of Tag 9F01, Tag 5F36 and Tag 9F53 as case required.

Example code for Android device

//Set the Tag in onTransInitBeforeGPO callback

emvHandler.setTlv(ConvertUtils.hexString2Bytes("5F36"), ConvertUtils.hexString2Bytes("00"));

emvHandler.setTlv(ConvertUtils.hexString2Bytes("9F01"), ConvertUtils.hexString2Bytes("000000000000"));

// Tag 9F53

emvHandler.setJcbContactlessTIP(new byte[]{(byte) 0x73, (byte) 0x00,(byte) 0x00});

UnionPay

Terminal Configuration

| Terminal type | POS |
| --- | --- |
| Terminal capability | Offline with online capability |
| Online Pin | Support |
| Signature | Support |
| Terminal PIN Bypass | Support |
| Terminal Support CDCVM function | Support |
| Terminal Contactless Floor Limit | 0.00 |
| Terminal Contactless CVM Limit | 300.00 |
| Terminal Contactless Trans Limit | 99999.99 |
| Terminal QPS limit | 300.00 |

Test AID & CAPK

AID

AID list:

A000000333010101

A000000333010102

A000000333010103

A000000333010106

AID detail for source code configuration:

{

"ddol": "9f3704",

"aid": "a000000333010101",

"appVerNum": "0020",

"asi": 0,

"contactlessCvmLimit": 30000,

"contactlessFloorLimit": 0,

"contactlessTransLimit": 9999999,

"floorLimit": 0,

"maxTargetPercent": 0,

"onlinePinCap": 1,

"tacDefault": "084000a800",

"tacDenial": "0010000000",

"tacOnline": "d84004f800",

"targetPercent": 0,

"threshold": 0,

"transLimit": 100000

}, {

"ddol": "9f3704",

"aid": "a000000333010102",

"appVerNum": "0020",

"asi": 0,

"contactlessCvmLimit": 30000,

"contactlessFloorLimit": 0,

"contactlessTransLimit": 9999999,

"floorLimit": 0,

"maxTargetPercent": 0,

"onlinePinCap": 1,

"tacDefault": "084000a800",

"tacDenial": "0000000000",

"tacOnline": "d84004f800",

"targetPercent": 0,

"threshold": 0,

"transLimit": 100000

}, {

"ddol": "9f3704",

"aid": "a000000333010103",

"appVerNum": "0020",

"asi": 0,

"contactlessCvmLimit": 30000,

"contactlessFloorLimit": 0,

"contactlessTransLimit": 9999999,

"floorLimit": 0,

"maxTargetPercent": 0,

"onlinePinCap": 1,

"tacDefault": "084000a800",

"tacDenial": "0010000000",

"tacOnline": "d84004f800",

"targetPercent": 0,

"threshold": 0,

"transLimit": 100000

}, {

"ddol": "9f3704",

"aid": "a000000333010106",

"appVerNum": "0020",

"asi": 0,

"contactlessCvmLimit": 30000,

"contactlessFloorLimit": 0,

"contactlessTransLimit": 9999999,

"floorLimit": 0,

"maxTargetPercent": 0,

"onlinePinCap": 1,

"tacDefault": "084000a800",

"tacDenial": "0010000000",

"tacOnline": "d84004f800",

"targetPercent": 0,

"threshold": 0,

"transLimit": 100000

},

CAPK

{

"arithInd": 1,

"capkIdx": 8,

"checkSum": "ee23b616c95c02652ad18860e48787c079e8e85a",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "b61645edfd5498fb246444037a0fa18c0f101ebd8efa54573ce6e6a7fbf63ed21d66340852b0211cf5eef6a1cd989f66af21a8eb19dbd8dbc3706d135363a0d683d046304f5a836bc1bc632821afe7a2f75da3c50ac74c545a754562204137169663cfcc0b06e67e2109eba41bc67ff20cc8ac80d7b6ee1a95465b3b2657533ea56d92d539e5064360ea4850fed2d1bf",

"rid": "a000000333"

}, {

"arithInd": 1,

"capkIdx": 2,

"checkSum": "03bb335a8549a03b87ab089d006f60852e4b8060",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "a3767abd1b6aa69d7f3fbf28c092de9ed1e658ba5f0909af7a1ccd907373b7210fdeb16287ba8e78e1529f443976fd27f991ec67d95e5f4e96b127cab2396a94d6e45cda44ca4c4867570d6b07542f8d4bf9ff97975db9891515e66f525d2b3cbeb6d662bfb6c3f338e93b02142bfc44173a3764c56aadd202075b26dc2f9f7d7ae74bd7d00fd05ee430032663d27a57",

"rid": "a000000333"

}, {

"arithInd": 1,

"capkIdx": 3,

"checkSum": "87f0cd7c0e86f38f89a66f8c47071a8b88586f26",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "b0627dee87864f9c18c13b9a1f025448bf13c58380c91f4ceba9f9bcb214ff8414e9b59d6aba10f941c7331768f47b2127907d857fa39aaf8ce02045dd01619d689ee731c551159be7eb2d51a372ff56b556e5cb2fde36e23073a44ca215d6c26ca68847b388e39520e0026e62294b557d6470440ca0aefc9438c923aec9b2098d6d3a1af5e8b1de36f4b53040109d89b77cafaf70c26c601abdf59eec0fdc8a99089140cd2e817e335175b03b7aa33d",

"rid": "a000000333"

}, {

"arithInd": 1,

"capkIdx": 4,

"checkSum": "f527081cf371dd7e1fd4fa414a665036e0f5e6e5",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "bc853e6b5365e89e7ee9317c94b02d0abb0dbd91c05a224a2554aa29ed9fcb9d86eb9ccbb322a57811f86188aac7351c72bd9ef196c5a01acef7a4eb0d2ad63d9e6ac2e7836547cb1595c68bcbafd0f6728760f3a7ca7b97301b7e0220184efc4f653008d93ce098c0d93b45201096d1adff4cf1f9fc02af759da27cd6dfd6d789b099f16f378b6100334e63f3d35f3251a5ec78693731f5233519cdb380f5ab8c0f02728e91d469abd0eae0d93b1cc66ce127b29c7d77441a49d09fca5d6d9762fc74c31bb506c8bae3c79ad6c2578775b95956b5370d1d0519e37906b384736233251e8f09ad79dfbe2c6abfadac8e4d8624318c27daf1",

"rid": "a000000333"

}, {

"arithInd": 1,

"capkIdx": 128,

"checkSum": "a5e44bb0e1fa4f96a11709186670d0835057d35e",

"expireDate": "20201231",

"exponent": "010001",

"hashInd": 1,

"modulus": "ccdba686e2efb84ce2ea01209eeb53bef21ab6d353274ff8391d7035d76e2156caedd07510e07dafcacabb7ccb0950ba2f0a3cec313c52ee6cd09ef00401a3d6cc5f68ca5fcd0ac6132141fafd1cfa36a2692d02ddc27eda4cd5bea6ff21913b513ce78bf33e6877aa5b605bc69a534f3777cbed6376ba649c72516a7e16af85",

"rid": "a000000333"

}, {

"arithInd": 1,

"capkIdx": 9,

"checkSum": "a075306eab0045baf72cdd33b3b678779de1f527",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "eb374dfc5a96b71d2863875eda2eafb96b1b439d3ece0b1826a2672eeefa7990286776f8bd989a15141a75c384dfc14fef9243aab32707659be9e4797a247c2f0b6d99372f384af62fe23bc54bcdc57a9acd1d5585c303f201ef4e8b806afb809db1a3db1cd112ac884f164a67b99c7d6e5a8a6df1d3cae6d7ed3d5be725b2de4ade23fa679bf4eb15a93d8a6e29c7ffa1a70de2e54f593d908a3bf9ebbd760bbfdc8db8b54497e6c5be0e4a4dac29e5",

"rid": "a000000333"

}, {

"arithInd": 1,

"capkIdx": 11,

"checkSum": "bd331f9996a490b33c13441066a09ad3feb5f66c",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "cf9fdf46b356378e9af311b0f981b21a1f22f250fb11f55c958709e3c7241918293483289eae688a094c02c344e2999f315a72841f489e24b1ba0056cfab3b479d0e826452375dcdbb67e97ec2aa66f4601d774feaef775accc621bfeb65fb0053fc5f392aa5e1d4c41a4de9ffdfdf1327c4bb874f1f63a599ee3902fe95e729fd78d4234dc7e6cf1ababaa3f6db29b7f05d1d901d2e76a606a8cbffffecbd918fa2d278bdb43b0434f5d45134be1c2781d157d501ff43e5f1c470967cd57ce53b64d82974c8275937c5d8502a1252a8a5d6088a259b694f98648d9af2cb0efd9d943c69f896d49fa39702162acb5af29b90bade005bc157",

"rid": "a000000333"

}, {

"arithInd": 1,

"capkIdx": 36,

"checkSum": "e284c81aa9102f7b68ff0c8c74b5fa5a48d0ba59",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "d7cd44b5c19d27ef8053f8f0adaa6280b1536b26d527c4e5b56b86964070af3be27227ea1eadc3adc9c2c7d476f020f352c5c7458df1f5609dbca7a8c2633c4951505e4b408ba8bb2f5685226299dddef1bcd4ef1f45b1e27213b375bff237746fb30d782e2e3e60e33b49262af573acb3100feca17c8666f57a029c9a6da71625685c1bd7554452b8b6282a3a0f099f",

"rid": "501649FF20"

}, {

"arithInd": 1,

"capkIdx": 10,

"checkSum": "C88BE6B2417C4F941C9371EA35A377158767E4E3",

"expireDate": "20201231",

"exponent": "03",

"hashInd": 1,

"modulus": "B2AB1B6E9AC55A75ADFD5BBC34490E53C4C3381F34E60E7FAC21CC2B26DD34462B64A6FAE2495ED1DD383B8138BEA100FF9B7A111817E7B9869A9742B19E5C9DAC56F8B8827F11B05A08ECCF9E8D5E85B0F7CFA644EFF3E9B796688F38E006DEB21E101C01028903A06023AC5AAB8635F8E307A53AC742BDCE6A283F585F48EF",

"rid": "a000000333"

},

Note: CapkIdx is int type in above list. For example 239(int) = 0xEF(Hexadecimal)

Test cases

UPI Quickpass need to set the QPS limit, and enable the CDVM function.

Please set below configuration in the source code before you start the test, you can also refer to Nexgo SDK EMV sample code:

UnionPayTransDataEntity unionPayTransDataEntity = new UnionPayTransDataEntity();

unionPayTransDataEntity.setQpbocForGlobal(true);

unionPayTransDataEntity.setSupportCDCVM(true);

unionPayTransDataEntity.setSupportContactlessQps(true);

unionPayTransDataEntity.setContactlessQpsLimit("000000030000");

transData.setUnionPayTransDataEntity(unionPayTransDataEntity);

emvHandler2.emvProcess(transData, this);

Matching Currency

This part test cases need to set the currency code to 0156, please refer below source code.

transData.setCountryCode("0156");   //CountryCode

transData.setCurrencyCode("0156");   //CurrencyCode

Unmatching Currency

This part test cases need to set the currency code you want to support, for example 840, please refer below source code.

transData.setCountryCode("0840");    //CountryCode

transData.setCurrencyCode("0840");    //CurrencyCode